Purchase Specifications for a 
5-9 Nautical Mile Range Marine Lantern

Overview

This specification is for a solar powered 5-9NM marine lantern.

1.0 Light Characteristics

The lanterns light source will be high intensity and high-powered LED’s.

The lantern output shall be available in red, green, white, yellow and blue.

The lantern shall have a typical maximum available intensity (dependent on model) of up to:

- Red lantern output 620cd

- Green lantern output 690cd

- White lantern output 1150cd

- Yellow lantern output 570cd

- Blue lantern output 310cd

The lantern shall have a visible range of 5-9NM+ (AT @ 0.74).

The lantern shall have a horizontal output of 0-360 degrees.

The lantern shall have a vertical divergence of 5 or 10 degrees.

The lantern shall have up to 256 IALA recommended user-adjustable flash characteristics.

The lantern shall have multiple intensity settings.

The lantern shall come with GPS Synchronisation and Bluetooth® connectivity and as standard.

The lantern shall have the option of satellite connectivity and AIS type 1 and type 3 as an option.

2.0 Electrical Characteristics

The lantern shall have integrated circuit protection.

The lantern shall have a nominal voltage of 12V DC.

The lantern shall have an operating temperature range between -30 to 50°C.

3.0 Solar Characteristics

The lantern shall have four (4) monocrystalline solar modules.

Each module shall be up to 8.4 watts (33.6 watts in total).

4.0 Power Supply

The battery type shall be Gel Sealed Lead Acid (SLA).

The battery shall have a capacity of 12 or 24 Ah.

The battery shall be user-replaceable.

5.0 Physical Characteristics

The body of the lantern shall be manufactured from an aluminium chassis with UV-stabilised rubber corners and gaskets.

The top and base of lantern will be composite moulded.

The lantern lens shall be UV-stabilised polycarbonate or equivalent.

The lantern shall have a lens diameter of 98mm (3⅞ inches).

The lens design shall be LED optic.

The lantern shall have a mounting pattern using 3 x 4 hole 200mm bolt pattern.

The lantern shall have a height of up to 660mm (26 inches).

The lantern shall have a width of 234mm (9⅕ inches).

The lantern shall have a mass from 9.2kg (20.3lbs) dependent on model.

The lantern shall have a life expectancy of up to 15 years.

6.0 Options

The lantern shall be offered with the following options available from the manufacturer:

- IR programmer

- Satellite Monitoring and Control System

- AIS Type 1 and Type 3

- External ON/OFF switch and LED display

- External Battery Charging Port

- 5° and 10° lens

- Higher current setting for high sunlight regions

- 50mm pole mount adapter plate

7.0 Certifications

The lantern shall be IP68 waterproof.

The manufacturer shall be ISO9001:2015 certified.

The lantern shall meet CE EN61000-6-2:2005.

The lantern shall meet IALA E-200-1 Signal Colours.

8.0 Warranty

The lantern shall have a three (3) year warranty, excluding battery which will have a warranty of one (1) year.